TC-C35WS 3ERA-4

5MP Fixed Color Maker Bullet Camera

Lite Series

KEY FEATURES

• Metal+plastic housing

• Up to 5MP,3072×1728@30fps

• S+265/H.265/H.264B/H.264M/H.264H

• Color:0.002Lux@(F1.6,AGC ON),B/W:0Lux with IR

• IR range up to 50m

• 1×Built-in Mic

• Support microSD/microSDHC/microSDXC card,up to 512GB

• Support human/vehicle classification

• Operating conditions -35℃~65℃(-31℉~149℉), ≤95%RH

• DC12V±25%,Max. 7W, PoE:IEEE 802.3af,Max. 7.5W

• IP67

SPECIFICATIONS

| Camera |  |
| --- | --- |
| Image Sensor | 1/2.7"CMOS |
| Min. Illumination | Color:0.002Lux@(F1.6,AGC ON),B/W:0Lux with IR |
| Shutter Time | 1s~1/100000s |
| Day&Night | IR-cut filter with auto switch (ICR) |
| Smart IR | Yes |
| WDR | 120dB |
| S/N | >39dB |
| Angle Adjustment | 0~360°(P); 0~90°(T); 0~360°(R) |

| Lens |  |
| --- | --- |
| Lens Type | Fixed focal lens |
| Focal Length | 4mm |
| Lens Mount | M12 |
| Aperture Range | F1.6 |
| Aperture Type | Fixed |
| Field of View | 91°(H); 46.4°(V); 107°(D) |

| DORI Distance | Detect:80m Observe:31.5m Recognize:16m |
| --- | --- |

Identify:8m

| Illuminator |  |
| --- | --- |
| IR LEDs | 2 |
| IR Distance | Up to 50m |

| Wavelength | 850nm |
| --- | --- |
| White LEDs | 2 |
| Whitelight Distance | 15m |

| Compression Standard |  |
| --- | --- |
| Video Compression | S+265/H.265/H.264B/H.264M/H.264H |
| Video Bit Rate | 32k~6Mbps |
| Audio Compression | G.711A/G.711U/ADPCM_D |
| Audio Sampling Rate | 8kHz/32kHz/48kHz |

| Image |  |
| --- | --- |
| Max. Resolution | 5MP,3072×1728@30fps |
| Main Stream | PAL:25fps(3072×1728, 2688×1520, 2560×1440, 2304×1296, 1920×1080, 1280×720) NTSC:30fps(3072×1728, 2688×1520, 2560×1440, 2304×1296, 1920×1080, 1280×720) |
| Sub Stream | PAL:25fps(704×576, 704×288, 640×360, 352×288) NTSC:30fps(704×480, 704×240, 640×360, 352×240) |
| BLC | Yes |
| HLC | Yes |
| AGC | Yes |
| 3D DNR | Yes |
| White Balance | Auto/Semi-Auto/Manual/Sunshine/Nature Light/Warmlight Lamp/Daylight Lamp/ Filament Lamp/Lock White Balance |
| ROI | 8 rectangular static areas |
| OSD | Overlay of additional characters in 1 regions, 1 line, with a maximum of 48 characters or 16 Chinese characters per region; 16*16/32*32/48*48/64*64/96*96; Lattice |
| Privacy Mask | 4 black static areas |
| Defog | Yes |

| Feature |  |
| --- | --- |
| General Alarm | Motion Detection; Mask Alarm |
| Exceptions Alarm | Disk Full; Disk R/W Error; IP Address Conflict; Network Disconnection |

| Linkage Method | Trigger Recording; Trigger Capture; Notify Surveillance Center |
| --- | --- |
| Video Analytics | Human/Vehicle Classification; Tripwire; Perimeter |
| Rules of Behavior Analyse In 1 Scene | 2 |

| Network |  |
| --- | --- |
| ANR | Yes |
| Protocols | RTSP; HTTP; HTTPS; TCP; UDP; UPnP; ICMP; SNMP v1\v2\v3; DHCP; DNS; DDNS; Easy DDNS; NTP; TLS v1.2; QoS; IPv4; SSH; Unicast; Multicast; ARP; FTP |
| System Compatibility | P2P(Mutually exclusive with Profile G); ONVIF(Profile S/T/G) ; SDK; CGI; Milestone |
| Remote Connection | ≤2 |
| Client | Easy7 VMS |
| Moblie Client | EasyLive Plus |
| Web | IE10+; Firefox52+; Edge89+; Safari11+; Chrome57+ |

| Interface |  |
| --- | --- |
| Communication Interface | 1×RJ-45 (10/100 Base-T) |
| Built-in Mic | 1×Built-in Mic |
| Reset Button | Yes |
| On-board Storage | Support microSD/microSDHC/microSDXC card,up to 512GB |
| RTC | Yes |

General

| Web Client Language | 20 languages Simple Chinese/Traditional Chinese/English/Spanish/Korean/Italian/Turkish/Russian/ Thai/French/Polish/Dutch/Hebrew/Arabic/Vietnamese/German/Ukrainian/Portuguese/ |
| --- | --- |

|  | Albanian/Greek |
| --- | --- |
| Operating Temperature | -35℃~65℃(-31℉~149℉) |
| Operating Humidity | ≤95%RH |
| Storage Temperature | -35℃~65℃(-31℉~149℉) |
| Storage Humidity | 10%~95%RH(non-condensing) |

| Power Supply | DC12V±25%,Max. 7W; PoE:IEEE 802.3af,Max. 7.5W |
| --- | --- |
| Protection | Surge4000V; ESD6000V; IP67 |
| Boundary Dimensions | 161(L)68(W)66(H)mm |
| Package Dimensions | 215(L)95(W)120(H)mm |
| GrossWeight | 0.4458kg |

DIMENSIONS (Unit: mm)

Assembly diagram of Junction Box