TC-H324S 323IA 
PTZ Camera

Pro Series

KEY FEATURES 
Up to 2MP,1920×1080@30fps 
Color:0.001lux@ (F1.5,AGC ON),B/W:0lux with IR 
S+265/H.265/H.264B/H.264M/H.264H/Motion JPEG 
Optical zoom 23X 
IR range up to 150m 
Support microSD/microSDHC/microSDXC card,up to 512GB Support smart surveillance 
Support human/vehicle classification 
Operating conditions -30℃~65℃(-22℉~149℉), ≤95%RH

DC12V±10%, PoE:IEEE 802.3at 
IP66

SPECIFICATIONS

| Camera |  |
| --- | --- |
| Image Sensor | 1/2.8"CMOS |
| Min. Illumination | Color:0.001lux@ (F1.5,AGC ON),B/W:0lux with IR |
| Shutter Time | 1s~1/100000s |
| Day&Night | IR-cut filter with auto switch (ICR) |
| Smart IR | Yes |
| WDR | 120dB |
| S/N | 42dB |
| Lens |  |
| Focal Length | 5mm~115mm |
| Optical Zoom | 23X |
| Zoom Speed | <5s |
| Aperture Range | F1.5~F3.0 |
| Aperture Type | Auto(P-Iris) |
| Field of View | 55.8°~3.2°(H); 32.2°~1.8°(V); 61°~3.64°(D) |
| Focus Control | Auto/Semi-auto/Manual |
| Close Focus Distance | 1000mm~1500mm |
| DORI Distance | Detect:1351m Observe:536m Recognize:270m Identify:135m |
| Pan&Tilt |  |
| Pan Range | 0~360°(P) |

| Pan Speed | Manual Speed:0.1°～80°/s Preset Speed:80°/s |
| --- | --- |
| Tilt Range | -16°~90°(T) (Default -3°~90°(T)) |
| Tilt Speed | Manual Speed:0.1°～80°/s Preset:80°/s |
| 3D Position | Yes |
| Preset | 500 |
| Cruise | 16(up to 32 presets per cruise) |
| Scan | 8 |
| Pattern | 8,each with 600s memory or 1000 orders |
| Scheduled Task | Preset,Cruise,Scan,Pattern |
| North Point | Yes(Manual setting) |
| Power-off Memory | Yes |
| Illuminator |  |
| IR LEDs | 7 |
| IR Distance | Up to 150m |
| Wavelength | 850nm |
| IR Irradiation Angle | Yes |
| Compression Standard |  |
| Video Compression | S+265/H.265/H.264B/H.264M/H.264H/Motion JPEG |
| Video Bit Rate | 32Kbps~16Mbps |
| Audio Compression | G.711A/G.711U/ADPCM_D/AAC_LC |
| Audio Sampling Rate | 8kHz/32kHz/48kHz |
| Image |  |
| Max. Resolution | 2MP,1920×1080@30fps |
| Main Stream | PAL:25fps(1920×1080,1280×720,640×480,704×576) NTSC:30fps(1920×1080,1280×720,640×480,704×480) |
| Sub Stream | PAL:25fps(704×576,704×288,640×360,352×288) NTSC:30fps(704×480,704×240,640×360,352×240) |

| Third Stream | PAL:25fps(352×288) NTSC:30fps(352×240) |
| --- | --- |
| BLC | Yes |
| HLC | Yes |
| AGC | Yes |
| 3DNR | Yes |
| White Balance | Auto/Semi-Auto/Manual/Sunshine/Nature Light/Warmlight Lamp/Daylight Lamp/ Filament Lamp/Lock White Balance |
| ROI | 7 Dynamic Areas for Both Main and Sub Stream |
| OSD | 5 regions(Each region can have a maximum of 127 characters ,up to 1 line); 16*16/24*24/32*32/48*48/64*64/adaptable size; Vector/Lattice |
| Defog | Yes |
| Flip | Normal/Horizontal Mirror/Vertical Mirror/Inverted Flip |
| Regional Exposure | Yes |
| Regional Focus | Yes |
| Demist | Yes |
| Feature |  |
| General Alarm | Motion Detection; Mask Alarm; Alarm Input |
| Exceptions Alarm | Voltage Alarm; Disk Full; Disk R/W Error; Recording Exception; IP Address Conflict; MAC Address Conflict; FTP Server Exception ; Network cable disconnected |
| Linkage Method | Send Email; Trigger Sound; Trigger Recording; Trigger Capture; Notify Surveillance Center; Alarm Output |
| Video Analytics | Human/Vehicle Classification; Tripwire; Double Tripwire; Perimeter; Object Abandon; Object Missing; Heatmap; Video Abnormal; Audio Abnormal |
| Network |  |
| ANR | Yes |
| Protocols | RTSP; RTMP; HTTP; HTTPS; TCP/IP; UDP; UPnP; ICMP; IGMP; SNMP v1\v2\v3; DHCP; DNS; DDNS; Easy DDNS; NTP; SMTP; TLS v1.2; SSL; 802.1x; QoS; IPv4; IPv6; PPPoE; SSH; Unicast; Multicast; ARP; RTP; SRTP; FTP; SFTP |
| System Compatibility | P2P; ONVIF(Profile S/T/G/M); SDK; CGI; Milestone; RTSP; RTMP; HTTP PUSH |
| Remote Connection | ≤2 |

| User/Host | 32users, Level 4:Admin/Browse/Browse+Control/Browse+Control+Set |
| --- | --- |
| Client | Easy7 VMS |
| Mobile Client | EasyLive Plus |
| Web | IE10+; Firefox52+; Edge89+; Safari11+; Chrome57+ |
| Interface |  |
| Communication Interface | 1*RJ45 (10/100Base-T) |
| Audio I/O | 1*Input Impedance 1kΩ,2.0~2.4Vp-p; 1*Output Impedance 600Ω |
| Alarm I/O | 1*Switching; 1*Switching,Voltage≤DC24V,Current≤1A |
| Reset Button | Yes |
| On-board Storage | Support microSD/microSDHC/microSDXC card,up to 512GB |
| General |  |
| Web Client Language | 19 languages Simple Chinese/English/Traditional Chinese/Spanish/Italian/ Korean/Turkish/Russian/ Thai/ Polish/French/Dutch/ Hebraism/Vietnamese/Arabic/German/ Ukrainian / Portuguese/Albanian |
| Operating Temperature | -30℃~65℃(-22℉~149℉) |
| Operating Humidity | ≤95%RH |
| Storage Temperature | -30℃~65℃(-22℉~149℉) |
| Storage Humidity | 10%~95%RH(non-condensing) |
| Power Supply | DC12V±10%; PoE:IEEE 802.3at |
| Power Consumption | 7.15W(IR OFF) ; 18.5W(IR ON) |
| Protection | Surge4000V; ESD6000V; IP66 |
| Material | Metal+plastic |
| Boundary Dimensions | 161(D)276(H)mm |
| Package Dimensions | 260(L)260(W)465(H)mm |
| GrossWeight | 3.8kg |
| NetWeight | 2.1kg |

DIMENSIONS (Unit: mm)

Assembly diagram of Junction Box